NIGGLI: A microcomputer program to calculate NIGGLI values and determine corresponding magma types.

G. Tsambourakis and S.F. Youl

Division of Mineral Products, CSIRO

Port Melbourne, Victoria 3207, Australia

Abstract:	In this work we describe a new, user friendly FORTRAN microcomputer program called "NIGGLI.FOR" for calculating Niggli Values from the weight precentages of analysed oxide in a rock.   The program also performs a search through a database of magma types to characterise the rock chemistry by determining the corresponding magma type

Introduction

There are two ways of classifying rocks - mineralogical and chemical.  Although the mineral composition of a rock uniquely determines its chemical composition, the opposite is not true;   a given set of oxide percentages may allow for a variety of corresponding mineral assemblages, the correct assignment of which may require additional knowledge about rock provenance.  Many methods have been devised to recalculate the oxide percentages into a set of mineral molecules (the so called "norm system") and all can be traced to the work of Cross, Iddings, Pirsson and Washington (1902).

Perhaps the most important change to the rules proposed by Cross et al (1902) was that introduced by Niggli (1936), with later modifications by Burri (1959).

In the Niggli system the chemical classification is recast in terms of "Niggli Values" in order to delineate a series of "magma types" these magma types correspond to a rock classification with removal of the parameter describing texture.  The system is commonly used in the literature, not only in the comparative study of the chemistry of rocks, but also in a series of "variation diagrams" in which the various groupings of oxides are plotted against the silica content of the rock.

In the present work we describe a new, user friendly FORTRAN microcomputer program "NIGGLI" for calculating Niggli Values and magma type from the weight percentages of analysed oxided in a rock.   The program also performs a search through a database of magma types to characterise the rock chemistry.

Methodology

The calculation of the Niggli values is based on the principle of translating weight percentages into molecular proportions.   This is attained by dividing the weight percent value of every oxide analysed by its molecular weight to obtain a number designated as its molecular proportion.   A list of the oxides incorporated in this program, and the molecular weights used, are given in Table 1.   The molecular proportions are multiplied by 1000 to avoid handling fractions.

TABLE 1.	Oxides used and their molecular weights (in brackets)

_____________________________________________________________

SiO2 (60.0843)	TiO2 (79.8988)	Al2O3 (101.961278)

Fe2O3 (159.6922)	Cr2O3 (151.9904)	V2O3 (149.8812)

FeO (71.8464)	MnO (70.93745)	NiO (74.6894)

MgO (40.3044)	CaO (56.0774)	BaO (153.3264)

SrO (103.6194)	ZnO (81.3794)	CoO (74.9326)

Na2O (61.978936)	K2O (94.196)	 	Li2O (29.8814)

Rb2O (186.935)	Cs2O (281.81026)	P2O5 (141.944524)

F- (18.9984032)	H2O+ (18.01528)	H2O- (18.01528)

CO2 (44.0098)	SO3 (80.0642)	S (32.066)

___________________________________________________________________________

Niggli arranged the molecular proportions into four groups.

I.	Al2O3

II.	FeO + Fe2O3 (calculated as FeO) + MnO

III.	CaO

IV.	Na2O + K2O

Rarely analysed oxides were added to their respective group I-IV.

Burri (1956) proposed the following changes to the grouping of these oxides, and these have been adopted in NIGGLI program:

a)	Cr2O3, V2O3, Fe2O3, NiO and CoO are added to FeO, the first three oxides at a ratio of 1:2, the rest at a ratio of 1:1.

b)	BaO, SrO, Rb2O and Cs2O are added to K2O, the first two oxides at a ratio of 2:1, the others at a ratio of 1:1.

c)	Li2O is added to MgO at a ratio of 1:2.

The sum of the molecular proportions in the groups I-IV is recalculated to 100, and designated as al, fm, c and alk.

The molecular proportion figures of SiO2, TiO2, P2O5, and H2O+ are recalculated and designated as si, ti, p, and h.

Although the above values are sufficient for the chemical characterization of rocks, it is desirable to obtain more details on the two components alk and fm.  This information is obtained by the construction of the two ratios k and mg (Burri, 1956).

K2O	K+

k	=	-----------------	=	--------------------

K2O + Na2O	K+ + Na+

MgO		Mg2+

mg	=	----------------------	=	-------------------------------------

FeO+MgO+MnO	Fe3+ + Fe2+ + Mg2+ + Mn2+

If the degree of oxidation of Fe is important, the following ratio is also used:

2Fe2O3	Fe3+

w    =	---------------------	=	--------------------

2Fe2O3+FeO	Fe3+ + Fe2+

The quartz index qz (from the German "Qartz-zahl") provided further differentiation of rock types based on their saturation with respect of SiO2.  The value qz is calculated as si-si' where si' depends on the relative values of the other molecular proportions.

Case 1: a)	al  >  alk   and al  <  (alk + c)

si'  =  100 + 4 alk

b)	al  >  alk   and  al  >  (alk + c)

si'	=  100 + 4 alk  or, if access alumina is calculated as corundum,

si'	=	100 + 5 alk - al

Case 2:	al  <  alk

si'	=	100 + 3 al + alk

If	qz	│  0 then the excess SiO2 appears as free quartz and the rock contains highly silicated silicates.

If	qz	<   0 then the rock is quartz-free, with a paragenesis of poorly silicated silicates such as olivine etc.

The values si, al, fm, c, alk, k and mg are used for the formulation of a system of "magma types".  Magma type is an abbreviated designation to characterize the chemistry of a rock;   it does not represent any form of rock classification and is not a rock name.

Program operation

After the Niggli values have been calculated in accordance with the rules above, the program performs a search through a database of magma types, called "MAGMA.DAT".  The data used in this database were taken without amendment from Burri (1959).  The values of si, al, fm, c, alk, k and mg are compared with those in the data base a maximum of seven times or until one or more matches are found, using different detection limits on each cycle.  The detection limits were chosen after considerable testing of the program and are designed to achieve the smallest possible number of matches.  The program will accept as a match any magma type in which all variables fall within these limits.   It will also state how many matches it has found and provide a message if something is wrong with the input or if the data base has been accidently damaged.

NIGGLI is written in standard FORTRAN 77.   It has been tested and found to be fully functional using MS-FORTRAN V5.0, Lahey FORTRAN, VAX VMS FORTRAN, and SUN FORTRAN.

There are two ways to input data to NIGGLI:

a)	Directly from the keyboard.

At the prompt type NIGGLI.  With the initiation of the program, two questions will appear on the screen:

Sample name:

How many elements have been analysed?

After both questions are answered, the program will ask you to input the data.

Oxides and wt-% (A6, F8.3).

The oxides can be typed in any order, upper and lower case should be used, but subscripts are not permitted.

b)	Using the "NIGGLI.INP" file.

The NIGGLI.INP file contains the same information and typed in the same order as the input on screen.

After the data has been typed, store the file and type at the prompt:

NIGGLI<NIGGLI.INP

When the program completes all calculations the following message will appear on screen:

STOP - PROGRAM TERMINATED.

A file called "NIGGLI.OUT"  is created for storage of the results;  it can be either viewed on screen by typing:

Type NIGGLI.OUT.

or printed out by typing:

Print NIGGLI.OUT.

A typical printout is presented in Table 3.

Hardware requirements

The NIGGLI software package requires IBM compatibility, a minimum of 256Kb RAM and MS-DOS 3.00 or higher.

References

Cross, W., Iddings, J.P., Pirsson, L.V., and Washington, H.G. (1902).  A quantitative chemico-mineralogical classification and nomenclature of igneous rocks:  J. Geology.  10, 555-690.

Niggli, P. (1936a)   Schweiz.  Mim. Petr.  Mitt.  16, 295-317.

Niggli, P. (1936b).   Schweiz.  Min.  Petr.  Mitt.  16, 335-399.

Burri, C. (1956).   Schweiz. Min.Petr.Mitt.  36, 29-48.

Burri, C. (1959).   Petrochemische Berechnungsmethoden auf AEquivalenter Grundlage.  	Mineral.- Geotechn. Reihe, Birkhaeuser Verlag. (German).  This book has been translated into English by the Israel Program for Scientific Translations Ltd, Jerusalem 1965.

SAMPLE:Olivine Basalt

OXIDES	Wt-%	MOL Wt-%	MOL PROP.	1000xMOL.PROP.

SiO2	51.510	60.08430	.85730	857.29550

TiO2	1.120	79.89880	.01402	14.01773

Al2O3	17.520	101.96130	.17183	171.82990

Fe2O3	1.510	159.69220	.00946	9.45569

FeO	6.690	71.84640	.09312	93.11532

MnO	.150	70.93740	.00211	2.11454

MgO	6.740	40.30440	.16723	167.22740

CaO	8.780	56.07940	.15656	156.56370

Na2O	3.340	61.97894	.05389	53.88927

K2O	.770	94.19600	.00817	8.17444

P2O5	.310	141.94450	.00218	2.18395

H2O+	.490	18.01520	.02720	27.19925

H2O-	.150	18.01520	.00833	8.32630

_______

TOTAL	99.080

NIGGLI VALUES

***************

al	fm	c	alk

25.58	41.88	23.30	9.24

si	k	ti	p	h	mg	w

127.61	.13	2.09	.33	4.05	.59	.17

"Quartz zahl" qz =  -9.35

MAGMA TYPE

*************

Magma of the Calc-Alkali series.

Gabbro-dioritic magma (femic, neutral to basic, mostly alkali-poor

and c-normal).

Name	si	al	fm	c	alk	k	mg

Normal gabbro-dioritic	130.	23.0	44.0	22.5	10.5	.20	.50

Availability

To obtain an executable copy of NIGGLI please send send a cheque Aus $20  addressed to CSIRO, to cover the cost (disk, packaging and postage).