DOCUMENTATION FOR PROGRAM SEARCH

Program SEARCH is used to search the ICDD-JCPDS database to find

known phases matching a diffraction pattern from an unknown

materials, which may contain one or more phases.

When running program SEARCH, it is first necessary to set up the

search by specifying the peak positions and the parameters which

govern how the search will be performed. The READ command may

be used to read in a peak/data file. As an alternative to supplying

peak positions from a file, peak positions may be input using the

EDIT command, which invokes the peak list editor. If SEARCH has

previously been run, the previous peaks and parameters may be

reloaded using command LOAD.

Search parameters may be modified using commands SET, STARTFILE,

and TITLE and displayed using command SHOW.

Peak positions and intensities may be modified using the EDIT command.

Units for peak positions may be d-spaces or two-theta angles, as

selected using the WAVELENGTH command.

The inclusion of deleted patterns may be set using command DELETED.

After the search parameters have been set, the search is performed

using command SEARCH, which is equivalent to entering commands

REDUCE, MATCH and REPORT. ICDD-JCPDS entries which have been

selected as potential "hits" are written onto a file which may be

read by the LOGIC program. Command LOGIC starts this program and

passes the file of "hits."

A search consists of four steps:

1  Setting the scope of the search -- restricting the

search to the entries in a ICDD-JCPDS subfile or by using

chemistry to constrain the search (command STARTFILE);

2  Reducing the extent of the search by dropping entries that

are unlikely to be "hits" (command REDUCE):

-- by eliminating entries that do not have at least one strong

peak matching one of the observed strong peaks ["Required Lines"];

-- by eliminating entries that have a strong peak in reqions

of the observed pattern where no significant intensity

is present ["Forbidden Lines"];

3  Matching the remaining ICDD-JCPDS entries against the list of

observed peaks -- ICDD-JCPDS entries with too many unmatched peaks

or not matching a sufficient number of observed peaks are

dropped; the remaining entries are written as a user subfile

(command MATCH);

4  Evaluating the the "goodness of fit" between each

remaining entries and the observed pattern -- optionally,

the remaining entries may be ranked and the entries with the

best fits may also be written as a user subfile (command REPORT).

The SEARCH command performs steps 2, 3 and 4 and thus is equivalent

to using the REDUCE, MATCH and REPORT commands sucessively.

These steps must be performed in order, but any step may be omitted.

===> COMMAND  DCL

Spawns a subprocess to execute DCL commands, type LOGOUT to

return back to SEARCH loss of the current work.

This command is useful for quick DCL "errands" such as answering a

mail message without exiting the program.

===> COMMAND  DELETED

Some of the entries in the database are marked as "deleted" by the

editors of the database. If these entries are included in the database

(using the ENCODE program, PDF1) it is possible to select that these

entries will be included or excluded from the operations of this

program.

This command causes logical name PS_DELETED to be set so that

subsequent operations with the program and with other programs in the

Powder Suite package will use the same option. The default mode for

the program is DELETED/EXCLUDE. However, if desired the PS_DELETED

logical name may be predefined (see customization instructions) to change

the default.

---  Qualifiers

/INCLUDE

Specifies that deleted entries will be included in the logical operations.

/EXCLUDE

Specifies that deleted entries will be not be included in the logical operations.

===> COMMAND  EDIT

Used to edit the list of peaks. Peaks may be added, deleted or changed.

This operation must be done interactively. Note that intensities may

be input with an arbitrary scale, but will be normalized internally

before use.

===> COMMAND  EXIT

Type EXIT (which can be abbreviated to EX) to exit from the program.

Alternately, pressing Control Z (^Z) will also cause the program

to exit. Either causes the current search information to be written

onto a load file (see commands SET/NAME and LOAD.)

When the load file is written, logical device name SEARCH_FILE

is defined as the name of the load file. If program SEARCH is

restarted in the same login session, the file defined by

SEARCH_FILE (if any) may be read.

===> COMMAND  FILE

Allows a series of commands to be input to the program from an external

file. The file should be prepared before starting the program, or using

the DCL command to invoke an editor.

---  Parameter

file_name

A single file may be specified on the command line containing the name

of the file with commands. If the file is not specified, a prompt will be

given to request a file name.

If no file extension is supplied, the default extension is .CMD

===> COMMAND  IGNORE

This command is used to display and modify the range of angles/d-spaces

which are used when searching the ICDD-JCPDS database. The ranges that have

been set to be ignored are called "excluded regions." Peaks in a

ICDD-JCPDS entry which are located in an excluded region are ignored,

e.g. their presence will not disqualify the entry in the REDUCE step,

nor will they be counted as unmatched peaks. Similarly, observed peaks

which fall inside an excluded area are not used for searching and are

not counted toward the percentage of matched peaks.

Two common applications which require use of excluded regions are:

(1) to insure that only peaks on ICDD-JCPDS entries within the observed

two-theta region are used -- the regions below and above the measured

diffraction pattern should be excluded. (2) to account for interference

from known or identified phases. For example, if silicon is used as an

internal standard, the silicon peaks could obscure a peak from an

unknown phase. Thus, the regions immediately surrounding the silicon

peaks should be treated as unknown and thus should be excluded from

consideration.

Qualifier /EXCLUDE is used to add or increase the size of an

excluded region and /INCLUDE is used to reset a region

so that it is no longer excluded. Qualifier /CARD is used to

exclude all peaks in a ICDD-JCPDS entries above a threshold intensity and

Qualifier /DISPLAY is used to display the current list of excluded

regions.

When a raw datafile is read into this program, regions outside the range

of the collected data are excluded. If a list of peaks is read from

a peak file without a datafile, the regions below the lowest angle

and above the highest angle are excluded. These excluded ranges may be

extended using /EXCLUDE and reduced using /INCLUDE.

/DISPLAY

Qualifier /DISPLAY produces a listing of the current "excluded regions".

If this qualifier is combined with other qualifiers, the excluded regions

will be displayed after any changes.

/CARD=jcpds_card

/CARD

Qualifier /CARD excludes regions surrounding a ICDD-JCPDS entry. If

supplied, jcpds_card is the 5 or 6 digit ICDD-JCPDS set and entry number

(e.g. /CARD=110011). The width of the region around each peak may be

specified using qualifier /WIDTH and the intensity threshold for

ignored peaks may be specified using /IMIN.

Qualifier /CARD cannot be used with /INCLUDE or /EXCLUDE.

/IMIN=min_inten

/IMIN specifies the minimum intensity for ICDD-JCPDS peaks which will be

"excluded". Peaks with relative intensity below min_inten (in units of

% of the most intense peak) will not be used.

/INCLUDE=position_list

/INCLUDE

Specifies a list of central points of regions which will be reset so

that these regions will no longer be "excluded" from the search. The

the length of each region will depend on the value of DEL_2THETA

(see /WIDTH.)

More than one position may be specified by enclosing the list in

parentheses and separating the points by commas, e.g.

/INCLUDE=4.0 or

/INCLUDE=(4.0,6.0)

The units for position_list will be d-spaces in Angstrom, unless

a wavelength has be set, in which case the units will be degrees

two-theta at the current wavelength.

Note that if /CARD has been used previously, all peaks that lie

in the region to be included will be no longer be excluded.

Qualifier /INCLUDE cannot be used with /CARD or /EXCLUDE.

/EXCLUDE=position_list

/EXCLUDE

Specifies the central point in the region to be "excluded" from the

search. The the length of each region will depend on the value of

DEL_2THETA (see /WIDTH.)

More than one position may be specified by enclosing the list in

parentheses and separating the points by commas, e.g.

/EXCLUDE=4.0 or

/EXCLUDE=(4.0,6.0)

The units for position_list will be d-spaces in Angstrom, unless

a wavelength has be set, in which case the units will be degrees

two-theta at the current wavelength.

Qualifier /EXCLUDE cannot be used with /INCLUDE or /CARD.

/WIDTH=D_2THETA

D_2THETA defines the half width to be allowed around a position

specified using the /EXCLUDE or the /INCLUDE qualifiers or the

ICDD-JCPDS peaks from a entry specified using the /CARD qualifier. The

units for D_2THETA are degrees two-theta for Cu K alpha 1

radiation.  D_2THETA may be a floating point number.

===> COMMAND  LOAD

This command is used to reload the output file ("load file") from

a previous run of SEARCH so that parameters may be easily

changed. Thus the search run may quickly repeated with a new set of

parameters, allowing the widening of a search that failed to find

any matching phases or narrowing of a search that yielded too many

unlikely matches.

The "search name" (see command SET /NAME) is set as the name of

the LOAD file, after dropping the file extension, if one has been

supplied.

---  Parameter

file_name

A single file may be specified on the command line containing the name

of the file to be read. If the file is not specified, a prompt will be

given requesting a file name to read.

The default file extension for the "load file" is .SEA_IN.

===> COMMAND  LOGIC

This command causes the SEARCH run to be terminated and program

LOGIC to be started. The results from the most recent search will

be passed to program LOGIC. The program will save the current SEARCH

parameters on a load file for future use before transferring control

to program LOGIC.

===> COMMAND  MATCH

This command causes the program to invoke the MATCH step in the

search procedure. In this step peaks in the observed pattern are

compared line-by-line to the peaks in each possible hit remaining

after the REDUCE step (or in the starting subfile, if the REDUCE

step will be omitted). If the positions of two peaks (as two-theta

values for CuK alpha 1) agree within DEL_2THETA, the peaks are

considered to "match" (see SET/ERROR).

For a JCPDS-ICDD entry to be retained by the MATCH step as a "hit"

all but DB_max_miss of ICDD-JCPDS peaks above DB_min_int percent

relative intensity must be matched (see SET/DB_IMIN and SET/MAXDBMISS).

Also, at least OBS_min_lines percent of the lines above OBS_min_int

percent relative intensity must be matched (see SET/MATCHOBS and

SET/OBS_IMIN.

===> COMMAND  QUIT

Type QUIT to exit from the program without writing a load file or

updating logical name SEARCH_FILE.

===> COMMAND  READ

Reads a list of peak positions and optionally a raw data file. Peaks

in the peak list that are assigned to the K alpha 2 wavelength will be

ignored all others will be converted to d-spaces.

The "excluded regions", the ranges of d-spaces which are ignored

during searching, are reset by the READ command. (See command IGNORE

for more information about excluded regions). If peaks are input

without a data file, the regions of d-spaces outside the range of

peaks will be excluded. If a diffractogram is input as well, the regions

outside the range of collected data will be excluded rather than the

range of the data points (the collected data will usually cover a

signifcantly larger region than the range of peaks). If the

diffractogram will be used for FOM computations (see SET/FOM and

REPORT), the background should be subtracted, prior to reading it in

to SEARCH.

The search name is defined from the peak file name (including

the device and directory if specified), but the file extension is

dropped. This file name is used to define the names of the "load

file," which is used for temporary storage of the current

SEARCH settings; the "search output file," which will contain

a user subfile created with the matches from the search; a

"search report," if command REPORT is used to prioritize the

matches; and a "starting subfile," if the STARTFILE command is

used to spawn program LOGIC to create a subfile to narrow down the

search using chemistry. See information on the SET /NAME

subcommand, for more information on file naming.

---  Parameter

file_name

A single file may be specified on the command line containing the name

of the file to be read. If the file is not specified, the program will

prompt for a file name to read.

It is usually preferable to supply the file name without a file

extension, as the default extension will then depend on the file

format. This improves the speed that the program has available

when searching for the file.

The subroutines which read data and peak files, (subroutines

READDATA and READPEAK respectively), will attempt to determine

the format of the file from the file contents and the extension.

---  Qualifier

/FORMAT=fileform

This optional qualifier may be used to specify the file format that will

be used for reading the file.

In most cases it is not necessary to specify the file format, as the

program will discern the file format from the file. However, in certain

cases where problems are encountered, it may be necessary to use the

format option.

===> COMMAND  REDUCE

This command causes the program to reduce the number of ICDD-JCPDS

entries that will be considered during the search by dropping entries

that are unlikely to be "hits" by performing two screening steps:

entries are eliminated if they do not have at least one strong

peak matching one of the observed strong peaks ["Required Lines"];

entries are eliminated if they have a strong peak in reqions

of the observed pattern where no significant intensity

is present [Forbidden Lines];

The screening steps improve the speed of searching the database because they

use the bitmap indices to eliminate rapidly from consideration entries that

cannot match the observed pattern.

The "Required Lines" screening step is governed by variables

Req_Index, R_Imin and DEL_2THETA, which are changed using command

SET (see qualifiers /R_INDEX, /Rmin and /Error).

This step looks up each peak in the observed pattern above

R_Imin% relative intensity in a peak index file selected by Req_Index.

The peak positions in observed pattern and database must be within

DEL_2THETA to match. Thus, patterns which do not have an observed peak

matching some peak in the JCPDS-ICDD database within DEL_2THETA will be

eliminated from further consideration as a potential match.

This screening step may be skipped, if desired, by setting Req_Index

to a value of 100 or greater.

The "Forbidden Lines" screening step is governed by variables

Forbid_Index, F_Imin and DEL_2THETA, which are changed using command

SET (see qualifiers /F_INDEX, /Fmin and /Error).

This step elminates from further consideration entries in the JCPDS-ICDD

database that have strong peaks where no intensity has been observed. The

peak index selected by Forbid_Index is searched for peaks in the regions

of the observed pattern where there are no peaks above F_Imin/% relative

intensity within DEL_2THETA and that have not been "excluded".

Any JCPDS-ICDD entry with a peak in one of these regions will be

eliminated from further consideration as a potential match.

This screening step may be skipped, if desired, by setting Forbid_Index

to a value of 100 or greater.

The REDUCE command need not be used, but if it will be used it must

be specified before MATCH (or REPORT, if MATCH will also be omitted.)

Command SEARCH performs the REDUCE, MATCH and REPORT steps

sequentially.

===> COMMAND  REPORT

Used to print a report on the matches from the previous search and/or

creates a graph with the observed pattern and likely matching patterns.

If the MATCH step was not performed first, the report may be run

using all entries in the current subfile (or those remaining after

removing entries in the REDUCE step). Note that the REPORT step

is usually rapid, but may take several minutes if more than a

thousand entries will be considered.

For each entry, if two or more peaks match the observed pattern,

three or four figures of merit (FOM) are computed based on how

close the patterns match. The angle FOM reflects the descrepancies

between matching observed and ICDD-JCPDS peak positions; the intensity

FOM reflects the descrepancies between matching observed and ICDD-JCPDS

peak intensities; a full-pattern FOM is computed if a raw data file

is input to this program; the overall FOM is computed from the

above FOM values.

The formula used for the overall FOM depends on the setting for

Calc_type (see command SET/FOM).

Following the computation of the FOM values and the optional sort

step, it is possible to display stick diagrams for the matching patterns

along with the observed peaks or the observed diffractogram for each pattern.

Patterns will be displayed by order of FOM, if sorted.

The graphical display shows the observed peaks or diffractogram and

an adjustable number of "hits." At the bottom of the page, the excluded

regions of the pattern and the "forbidden" regions of the pattern are

shown as two lines. If the graphics device supports display of more than

one color on the screen. Different colors will be used to show excluded

peaks. Otherwise, different line styles will be used.

---  Qualifiers

/PRINT

Causes the report to be printed rather than displayed first on the

terminal.

===> COMMAND  SEARCH

This command causes the program to invoke the search procedure.

The search may be performed interactively or in batch mode.

If batch mode is selected, a load file will be written,

a batch job will be submitted and the program will then exit.

The SEARCH command performs the equivalent of the following

three commands: REDUCE, MATCH and REPORT

---  Qualifiers

/BATCH

This option causes the search to be performed in batch mode.

/INTERACTIVE

This option causes the search to be performed interactively.

/PRINT

Causes the report to be printed rather than displayed first on the

terminal. Ignored if used with the /BATCH option.

===> COMMAND  SET

This command is used to set parameters which govern the search.

If SET is called with one or more qualifiers, the specified

parameters are updated and no prompts are generated. However, if

SET is used with no qualifiers, prompts are generated which allow

all parameters to be updated.

When the search is performed (commands REDUCE, MATCH and REPORT),

peak positions from each appropriate entry in the ICDD-JCPDS database

are compared to positions from the list of peaks in the observed pattern.

Two peaks are considered to "match", if the two-theta values

agree within DEL_2THETA (see /ERROR).

In the MATCH step, observed and database entries are compared

peak-by-peak. If no observed peak matches for a ICDD-JCPDS peak and

the relative intensity of that peak is above DB_min_int percent,

(see /DB_IMIN), the peak is counted as a "miss." If more than

DB_max_miss ICDD-JCPDS peaks are not "missed"(see /MAXDBMISS),

the entry will be rejected.

Also in the MATCH step, the number of peaks in the observed pattern

which have been matched to ICDD-JCPDS are also counted. If fewer than

OBS_min_lines percent of the total observed peaks above OBS_min_int

percent relative intensity are matched, the entry is rejected.

Note that an observed peak may be matched by several ICDD-JCPDS lines,

and a ICDD-JCPDS line can be matched by several observed peaks.

The speed of the search is improved through three optional steps.

The first is by restricting the search to a ICDD-JCPDS or user subfile

created in program LOGIC (see command STARTFILE). The other two

methods utilize the strong peak indices to eliminate entries quickly

which have strong peaks where no intensity is observed (see /F_INDEX

and /Fmin, below) and/or to restrict the search to entries having at

least one strong line matching the an observed line in the pattern

(see /R_INDEX and /Rmin, below). The strong peak choices are utilized

in the REDUCE step, thus these parameters are ignored unless either

command REDUCE or SEARCH is used.

---  Qualifiers

/Error=DEL_2THETA

DEL_2THETA defines the allowed discrepancy between peak positions

in the observed pattern and those in the database used in the

MATCH and REPORT steps.

These steps match the peak positions using peak positions transformed

to angles for Cu K alpha 1 radiation. Thus, DEL_2THETA defines a

discrepancy in units of degrees two-theta for Cu K alpha 1 radiation,

regardless of the wavelength mode (see command WAVELENGTH) used for

input of the peak positions.

DEL_2THETA will typically be in the range 0.05 to 0.20 for most

searches. For materials where the lattice dimensions may vary (i.e.

solid solutions), or where either the observed peaks or the database

are presumed inaccurate, larger values may be needed.

/R_INDEX=Req_Index

/Rmin=R_Imin

Req_Index and R_Imin determine how the "Required Lines" screening

step in the REDUCE command is performed. In this step, entries from the

the starting subfile (see command STARTFILE) are dropped from

consideration unless one of the observed peaks matches a peak from

a peak index.

This screening step may be skipped, if desired, by setting Req_Index

to a value of 100 or greater.

Each non-excluded peak in the observed pattern with intensity above

R_Imin% of the strongest non-excluded observed peak will be checked

in this step to see if matches within DEL_2THETA of a peak in the

peak index selected by Req_Index. Thus, this step will reject from

further consideration as a potential match patterns that do not have at

least one observed peak matching some peak in the JCPDS-ICDD database.

The allowed values for R_Imin are between 0 and 99. The allowed values

for Req_Index are between -1 and 100 and have the following meaning:

100 Skip the "Required Line" step.

-1  Use the "Three Strongest" Peak Index file.

n   Use the "n% and Above" Peak Index file (n = 0 to 99).

Note that if the requested index file is not available,

the next smallest file (at 10% increments) will be used.

R_Imin will typically be a fairly small number, 5-10%. For

patterns with many peaks, where the REDUCE step does not eliminate

many entries from consideration, it may be desirable to raise R_Imin

up to the range of 25% or higher.

Req_Index will typically be 70% or -1 ("Three Strongest") so that

only patterns with reasonably strong matching peaks will be located.

If there is fairly strong preferred orientation or other errors which

increase the likelyhood that strong peaks will not match between patterns,

it may be necessary to use a lower value, 30% or 0%, for Req_Index.

Likewise, when the range of excluded regions in the search becomes large,

the probability that the excluded regions will contain all of the strong

peaks in the desired matching phase and thus the pattern will not be found.

The likely situations where this may occur is when data were collected only

over a small region of two-theta or when looking for a minor phase, where

a large number of peaks in the pattern have already been identified.

In these circumstances, if finding appropriate matches becomes difficult,

lower Req_Index or even turn off this step (by setting Req_Index to 100).

/F_INDEX=Forbid_Index

/FWidth=FWHM

/Fmin=F_Imin

Forbid_Index and F_Imin determine how the "Forbidden Lines" screening

step in the REDUCE command is performed. In this step, entries from the

the starting subfile (see command STARTFILE) are dropped from

consideration if there is a peak in a peak index that falls in an area

of the pattern where no peaks greater than F_Imin% are observed.

In this step, the pattern is broken up into approximately 260 regions.

Each region is checked to ensure that there are no peaks greater than

F_Imin% of the strongest non-excluded observed peak within FWHM

degrees and that the region has not been marked as excluded. If there

are no peaks and no excluded regions, the peak index selected by

Forbid_Index is checked for peaks in this region. Any peak found

in the region will eliminate the entry from further consideration.

The pattern is processed using using peak positions transformed

to angles for Cu K alpha 1 radiation. Thus, FWHM defines a width

in units of degrees two-theta for Cu K alpha 1 radiation,

regardless of the wavelength mode (see command WAVELENGTH) used for

input of the peak positions.

The allowed values for F_Imin are between 0 and 99. The allowed values

for Forbid_Index are between -1 and 100 and have the following meaning:

100 Skip the "Forbidden Line" step.

-1  Use the "Three Strongest" Peak Index file.

n   Use the "n% and Above" Peak Index file (n = 0 to 99).

Note that if the requested index file is not available,

the next smallest file (at 10% increments) will be used.

F_Imin will usually be a fairly small number, 1-4%, and

Forbid_Index will typically be in the range of 30-50%. This implies that

a JCPDS-ICDD peak in the range of 30-50% will be observed in the

experimental pattern at least at the 4% range. This may not be true

if the specimen has extreme preferred orientation or is composed of several

phases where no phase constitutes a majority of the sample or if the

JCPDS-ICDD pattern has preferred orientation. In these cases, where the

REDUCE step leaves no reasonable materials as potential "hits", one

should lower F_Imin, perhaps as low as 0% and/or raise Forbid_Index

to 70-90% or -1. In extreme cases, for example, where one want to find

matches to JCPDS-ICDD patterns where stong peaks may be completely missing

from the observed pattern, it may be necessary to skip this step (by setting

Forbid_Index to 100.) For materials where the REDUCE step does not eliminate

enough entries from consideration, it may be desirable to raise F_Imin up

to the range of 10-15% and set Forbid_Index to 30%.

/MAXDBMISS=DB_max_miss

/DB_IMIN=DB_min_int

In the MATCH step, the JCPDS-ICDD entries are compared one at at

time to the observed peaks. The number of peaks in the JCPDS-ICDD

pattern that must be matched for the entry to be given further

consideration is dictated by the values of DB_max_miss and

DB_min_int. All JCPDS-ICDD entries that do not match this

criteron are removed from the list of possible "hits."

For a pattern to be considered a likely match for an entry one

expects most if not all of the strong peaks in the JCPDS-ICDD entry

to be observed. Weak peaks may not always be seen depending on the

relative amount of the phase and the quality of the sample and the

pattern. A pattern will be discarded if more than DB_max_miss peaks

above DB_min_int% relative intensity are not within DEL_2THETA

of an observed peak. Thus DB_min_int% determines the threshold

for what is considered to be an observable intensity level. If the

material is believed to be single phase, DB_min_int should be a

fairly larger number, 10-20%. But for identifying phases in a material

with many phases, it may be necessary to raise DB_min_int as high as

50%. Under circumstances, the observed peaks should match all of the

JCPDS-ICDD peaks and DB_max_miss should be 0 or at most 1. However,

any of the following conditions can lead to strong JCPDS-ICDD peaks

to be missing from an observed pattern: severe preferred orientation

in the observed (or database!) pattern; an impurity in the database

pattern (rare but sometimes occurs); peak shifts due to instrumental

errors.

In general it makes sense to start with small values for both

DB_max_miss and DB_min_int. If no appropriate "hits"

are found. It is advisible to raise first DB_min_int and then

DB_max_miss. Both values must be integers.

/MATCHOBS=OBS_min_lines

/OBS_IMIN=OBS_min_int

In the MATCH step, the JCPDS-ICDD entries are compared one at at

time to the observed peaks and patterns that do not match the observed

pattern are removed from further consideration. Entries are removed

unless at least OBS_min_lines percent of the lines above OBS_min_int

percent relative intensity are matched.

In general it makes sense to start with values in the range of 50% for

OBS_min_lines and 25% for OBS_min_int. Meaning that a database entry

must match at least half of the observed lines above 25% relative

intensity. If the MATCH step fails to find any likely "hits," it is

wise to lower OBS_min_lines and/or raise OBS_min_int so that fewer lines

need to be matched for a hit to be obtained. Lower OBS_min_lines first,

if a multiphase mixture is suspected.

If too many "hits" are found to review conveniently, it is

wise to increase OBS_min_lines and/or lower OBS_min_int. If a

multiphase mixture is suspected, lower OBS_min_lines first.

/MAXCARDS=MAX_hits

The MATCH step will be ended early if more than MAX_hits entries are

found in the search. This can be used to prevent "run-away" searches

which turn up too many entries to comfortably review. When the search

is performed interactively, the search can either be continued or

abandoned if the MAX_hits threshold is reached.

The default value for MAX_hits is 999. If this many entries are

located, it is likely that the criteria for the REDUCE and MATCH

are too broad and the run will need to be repeated. For very slow

computer systems or where CPU time is at a premium, it may be wise

to lower this value. Under most conditions the value of MAX_hits

is unimportant.

/SORT=Num_Sort

/SORT

/NOSORT

This controls sorting of entries by figure of merit (FOM) in the

REPORT (or SEARCH) command. If Num_Sort is positive, the step

will list the Num_Sort ICDD-JCPDS entries with the best FOM and a subfile

with these entries will be written. If Num_Sort is zero or /SORT

is specified without a number for Num_Sort the sort will be performed

but all "hits" from the MATCH (or REDUCE step, if MATCH is

skipped) will be listed in the report. Using a negative value for

Num_Sort or specifying /NOSORT prevents sorting, speeding the REPORT

step.

/FOM=Calc_type

Determines how the overall figure of merit (FOM) is computed in the

Report command.

If Calc_type is 1, differences between matching observed and

ICDD-JCPDS peak positions do not affect the overall FOM.

If Calc_type is 2, differences between observed and ICDD-JCPDS

peak intensities do not affect the overall FOM.

If Calc_type is 3, the full-pattern FOM formula is used for the

overall FOM. This option is appropriate only if a raw data file has been

input to the program and also requires that the background be subtracted

from the data before it is read into the program.

/NAME=search_name

This qualifier is used to to define the names that will be used

for files that will be created during the run. While this name may

be as long as 40 or more characters (VMS allows very long filenames),

for convenience, the name will typically be 4 to 10 characters. The

search name is subject to the normal VMS conventions governing

file names: all letters, numbers and some symbols (notably _ and -)

are allowed, but spaces and most other symbols are not allowed.

If files are to be written in a directory or disk other than the

current default, device specifications may be included in the

search_name. File extensions are allowed, but their use is

not recommended. If supplied, the file extension will be used for

naming the "load file" only.

The following files are created by program SEARCH:

The "load file" is used for temporary storage of the current

SEARCH settings. This file may be reread by a later run of

SEARCH and will also be used if SEARCH is run as a batch job.

The file name is taken from the search name. File extension

.SEA_IN is used, if none is specified in the search name.

The "starting subfile" is created if the STARTFILE command

is used to narrow down the number of entries to be considered

during the search by creating a user supplied subfile.

The file is written by program LOGIC. The subfile to be written

will be named using the current search name, however, the

device/directory will be changed to the scratch directory,

PS_TMP, the string "_SEARCH" will be appended to the

search name, and the file extension will be set as .USRPDF.

The "search output file"is a user subfile with the

matches (hits) from the search. This file may be read by

program LOGIC. The file will be named directly from the

search name; the file extension will be .USRPDF.

The "top hits output file"is a user subfile with

the best ranked matches from the SEARCH or REPORT

command if sorting has been requested. The file is the

same as "search output file" except _TOPxx is appended

to the search name (xx is the number of sorted entries).

===> COMMAND  SHOW

This command is used to display the parameters which govern the

search, as established by command SET.

===> COMMAND  STARTFILE

This command is used to select the entries in the ICDD-JCPDS database that

will be considered when performing the search. The entire database

may be searched or the search may be restricted to a smaller section

(subfile). A number of standard subfiles have been prepared by the

ICDD-JCPDS; alternately program LOGIC may be used to prepare a

user-defined subfile.

Use of a subfile (any option other than /SUBPDF=FULL) reduces

the number of entries that need be examined, which can greatly speed the

search. For example, if it can be assumed that no elements heavier

than iodine are present in the unknown, program LOGIC can be used to

eliminate approximately one-third of the entries in the database from

consideration. (This is performed using command

CULL/E_SP=(Xe,Lr)/OR

within program LOGIC.)

Program LOGIC is most commonly used to specify chemistry constraints.

To call program LOGIC from within SEARCH, enter command STARTFILE/LOGIC.

To read a subfile previously prepared by LOGIC and stored in a

user-defined file, use command STARTFILE/USERFILE=file_name to select

that file. The user-defined subfiles created at the end of SEARCH run

may also be used as a starting point.

Only one of the following three options may be specified.

---  Qualifiers

/LOGIC

Specify this qualifier to call program LOGIC to create a new

user-supplied subfile to be used in the search. When the LOGIC

program is exited, a subfile will automatically be written and

program SEARCH will be resumed.

The subfile to be written will be named using the current search

name, however, the device/directory will be changed to the scratch

directory, PS_TMP, the string "_SEARCH" will be appended to

the search name, and the file extension will be set as .USRPDF.

/USERPDF=file_name

Use a previously written user-supplied subfile with name file_name.

The default file extension is .USRPDF.

/SUBPDF=sub_file

Use this option to start from one of the standard subfiles, such as the

the inorganic subfile.

The available subfiles are:

FULL         all entries in the database

NULL         null table (a table without any entries)

INORGANICS   Inorganic materials

ORGANICS     Materials with a C-H bond

COMMON       commonly found materials or phases

MINERALS     Minerals, natural and synthetic

METALS       metals, alloys and related materials

NIST         patterns recorded by NIST (NBS)

FORENSIC     materials of forensic interest

ZEOLITES     zeolites and molecular sieves

DELETED      superceeded but not removed patterns (if any)

LOCAL        locally defined patterns (if any)

SUPERCON     superconductors and related materials

EXPLOSIVES   explosives and related materials

EDUCATION    educationally relevant materials

CEMENT       materials related to cement production

CORROSION    corrosion products and related materials

Only one subfile may be specified. To search two subfiles, (for

example the MINERALS and METALS subfiles) use program LOGIC

(see /LOGIC) to OR (ADD) together these two subfiles.

Note that the sub_file name may be abbreviated by only including

the initial letters as long as the name is uniquely specified. Also

note that "DELETED" patterns are not used unless the DELETED/INCLUDE

command is used to force inclusion of these patterns.

===> COMMAND  TITLE

Used to change the "search title." The only purpose of the search

title is to provide a mechanism for saving comments and other

information from a search in the "load file."

The format of this command is:

TITLE title

The title can be specified on the command line or will be prompted.

If the title is put on the command line, any lower case letters used

in the title will be converted to uppercase.

---  Examples

TITLE This is the new title for the search

Note that the title will be converted to all

capital letters.

TITLE

The user will be prompted to specify a title.

===> COMMAND  WAVELENGTH

Peaks positions may be input or output as d-spacings (in Angstrom) or

as two-theta (Bragg) angles corresponding to a given x-ray or neutron

wavelength. This command is used to select between d-spacings

and angles and to select a wavelength for the latter option.

The current mode of the program is displayed in the header line at the

top of the computer screen. The default when the program is started

is determined by logical name PS_WAVELENGTH. If this logical name

is not defined, the default is to operate in d-space mode. When

the WAVELENGTH command is used logical name PS_WAVELENGTH

is redefined and this becomes the new default mode.

---  Parameter

wave_length

A single wavelength value may be specified directly, where wave_length

is the x-ray or neutron wavelength in Angstrom. (The target_type will be

assumed to be SY for synchrotron radiation).

If 0.0 is used for the wave_length, the program will enter d-space

mode (equivalent to specifying (WAVELENGTH/NOTARGET).

If wave_length is used, /TARGET and /WTYPE should not.

---  Qualifiers

/TARGET=target_type

/TARGET

/NOTARGET

Specifies an x-ray target for conventional x-ray sources.

The choices for targets are:

Cu, Mo, Ni, Co, Fe, Cr, Ag, NU, or SY.

Specifying /TARGET without a target_type is equivalent to

/TARGET=CU. (Capitalization of the target type is ignored.)

If NU or SY (for neutron or synchrotron radiation) is used,

the program prompts for a wavelength.

For input/output using d-spacings specify /NOTARGET

The /TARGET option cannot be specified with a wave_length value.

/WTYPE=wave_type

wave_type may be either 1, 2 or 3 which determines

the exact value used for the wavelength specified with /TARGET.

The choices are:

1 to use K alpha 1 radiation

2 to use K alpha 2 radiation

3 to use the intensity weighted average of K alpha 1 and K alpha 2

radiation.

Specifying /WTYPE without a value for wave_type is equivalent

to specifying /WTYPE=1.

This qualifier is ignored if the radiation source has a single

wavelength, (e.g. /TARGET=NU or /TARGET=SY) or a

wave_length is specified.

---  Examples

WAVELENGTH/TARGET=CU/WTYPE=3

The wavelength will be set to the average of Cu K alpha 1 and

K alpha 2.

WAVELENGTH/TARGET

The wavelength will be set to Cu K alpha 1.

WAVE/NOTAR

Peaks positions, etc. will be displayed as d-spaces.

WAVE 0.0

Peaks positions, etc. will be displayed as d-spaces.

WAVE 2.5401

The wavelength will be set to 2.5401Angstrom.

WAVE

The user will be prompted to choose a wavelength or two use

d-spacings.

WAVELENGTH/TARGET=NU/WTYPE=3

The user will be prompted to enter a wavelength; the /WTYPE=3

qualifier will be ignored.